CONFINED SPACE - MCQ ASSESSMENT 25 Q - FINAL

| Candidate name |  |
| --- | --- |
| Company / ID No. |  |
| Date |  |
| Score | / 100 |

Instructions to candidates

- Answer ALL 25 questions. Each question has only ONE correct answer.

- Circle the letter (A, B, C or D) of the correct answer. Each correct answer scores 4 marks (total 100 marks).

- Pass mark: 60 % (15 correct answers out of 25).

- Time allowed: as advised by the instructor.

- Before a confined space entry is authorised, what is the FIRST question to ask under the hierarchy of control?

- Which PPE should the entrants wear?

- Can the work be done without entering the space?

- How many entrants will be needed?

- How long will the work take?

- Which of the following can be a confined space on an oil and gas facility?

- A control room designed for continuous occupancy

- A site office building

- A large crude oil storage tank entered through a manway

- An open pipe-rack walkway with normal access

- In a silo or hopper, what is "bridging" of stored free-flowing solids?

- A temporary walkway laid across the material

- A crust over a hidden void that can collapse when stood on

- A method of breaking up material from outside

- The ladder used to reach the silo bottom

- A detector with a catalytic (pellistor) LEL sensor reads 0 % LEL and 2 % oxygen inside a vessel. What does this mean?

- Safe — there is no flammable gas present

- Safe for a short entry of up to 10 minutes

- Safe after a bump test of the detector

- NOT safe — the oxygen level is deadly and the LEL reading cannot be trusted

- A vessel has been purged with nitrogen. What must happen before anyone enters?

- Air-purge (ventilate) it and test the oxygen at all levels

- Enter at once — nitrogen is not toxic

- Ventilate it with more nitrogen

- Add pure oxygen to freshen the air

- Which pairing correctly describes the main responsibility of the Site Controller and the Area Authority?

- Site Controller – safe state of the plant; Area Authority – operates the rescue winch

- Site Controller – keeps the entry log; Area Authority – enters the space first

- Site Controller – overall control of work and SIMOPS; Area Authority – safe state of the plant

- Site Controller – carries out the gas test; Area Authority – acts as attendant

- Which pairing correctly describes the main responsibility of the Performing Authority and the Area Operator?

- Performing Authority – controls the work party and toolbox talk; Area Operator – applies and verifies isolations

- Performing Authority – applies the isolations; Area Operator – chairs the SIMOPS review

- Performing Authority – acts as rescue team leader; Area Operator – signs as gas tester

- Performing Authority – keeps the plant register; Area Operator – authorises the permit

- In the hierarchy of controls used in the risk assessment, where does PPE come?

- It replaces the need for isolation

- It is only needed when the gas test fails

- It is the first and most effective control

- It is the last line of defence, used after other controls

- Which isolation method gives the highest level of protection against hazardous energy and process fluids?

- A single closed valve

- A warning tag on the valve

- Positive isolation (for example, blinding or disconnection)

- A verbal instruction to the control room

- Why is a control-room "stop" command or a DCS interlock NOT accepted as isolation of an agitator inside a vessel?

- It takes too long for the agitator to stop

- It can be defeated by a signal, a fault, an operator action or a system restart

- It can only be used during the day shift

- It must first be approved by the gas tester

- Which statement about a confined space opening (entry and exit point) is correct?

- Barrier it as soon as the cover is removed and keep it clear of hoses and cables

- Barriers are needed only at night

- Ventilation ducting may be laid across the opening

- The opening may be left unguarded during breaks

- Why must a filter (air-purifying) respirator never be used in an oxygen-deficient atmosphere?

- A filter is too heavy to wear inside the space

- A filter blocks radio communication

- A filter cannot create oxygen

- A filter only works in hot conditions

- Contact with an entrant is lost and cannot be restored by the primary or backup method. What should the attendant do?

- Enter the space to check on the entrant

- Wait until the end of the shift

- Send another worker in to check

- Raise the alarm and activate the rescue plan — do not enter

- Which statement about gas welding and cutting (hot work) inside a confined space is correct?

- The pre-entry gas test alone is enough

- It needs a hot work permit, continuous gas monitoring and a fire watch

- No extra permit is needed if the vessel is empty

- Oxy-fuel hoses may be left inside during breaks

- Why must chemicals never be mixed inside a confined space?

- Some mixtures give off toxic gas or heat (e.g. acid with sulphide gives H₂S)

- Mixing only makes the chemicals less effective

- Mixing is allowed if the ventilation is running

- Mixed chemicals cannot be removed from site

- If a gas detector alarm sounds while people are inside the space, what should happen first?

- Investigate the cause while people stay inside

- Switch off the alarm and continue work

- Evacuate the space first, then investigate

- Wait five minutes to see if the alarm stops

- A confined space entry permit has reached the end of its validity, or the working conditions have changed. What must happen?

- Work continues until the job is finished

- Work stops and entrants leave; the permit is revalidated or reissued and the space re-tested before re-entry

- The permit is extended verbally by the Performing Authority

- Only the attendant needs to be told

- Which rescue method is preferred, and when must it be planned?

- Entry rescue by the nearest colleague, planned after an emergency

- Entry rescue by the attendant, planned at the end of the shift

- Waiting for the casualty to recover, with no plan needed

- Non-entry rescue using the retrieval system, planned before the permit is issued

- Apart from the set refresher interval, when should confined space refresher training also be given?

- After an incident or near miss involving confined space entry

- Only when the person changes employer

- Never — initial training is valid for life

- Only when the person asks for it

- A worker has completed confined space training but has not been formally authorised. May they act as an attendant?

- Yes — the training certificate is enough

- Yes — if a supervisor is nearby

- No — only an authorised person may perform the role

- Yes — for jobs shorter than one hour

- Which is an additional competency for personnel entering confined spaces to support integrity (inspection) work?

- Operating mobile cranes

- Driving heavy goods vehicles

- Preparing the site catering plan

- Recognising pyrophoric scale, NORM scale and hazardous deposits

- How should contractor training and competency records be checked?

- Accepted verbally from the contractor supervisor

- Verified at mobilisation, not accepted on assurance

- Checked only after an incident

- Not checked — contractors keep their own records

- Which electrical equipment is suitable for use inside a confined space where a flammable atmosphere could occur?

- Standard mains-voltage power tools

- Domestic extension leads and plugs

- Certified explosion-protected (Ex) equipment, preferably low-voltage or battery powered

- Any equipment, provided the ventilation is running

- For a vertical entry through a top manway, which arrangement provides retrieval and fall protection?

- A rope tied around the entrant's waist

- A safety helmet and gloves only

- The attendant holding the entrant's hand while they climb down

- A full body harness connected to a tripod or davit with a winch or self-retracting lifeline

- Which statement about ladders used for entry and exit is correct?

- They must be secured, inspected before use and kept clear so entrants can leave quickly

- They may be removed once the entrants are inside

- Tools should be carried in the hands while climbing

- Any ladder found on site is acceptable

— End of paper —

CONFINED SPACE - MCQ ASSESSMENT 25 Q - FINAL

ANSWER KEY (instructor use only)

| Q | Answer | Q | Answer | Q | Answer | Q | Answer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | B | 8 | D | 15 | A | 22 | B |
| 2 | C | 9 | C | 16 | C | 23 | C |
| 3 | B | 10 | B | 17 | B | 24 | D |
| 4 | D | 11 | A | 18 | D | 25 | A |
| 5 | A | 12 | C | 19 | A |  |  |
| 6 | C | 13 | D | 20 | C |  |  |
| 7 | A | 14 | B | 21 | D |  |  |

Scoring: 4 marks per correct answer; total 100 marks.

Pass mark: 60 % = 15 correct answers (60 marks) or more.

Full answers

1. B — Can the work be done without entering the space?

2. C — A large crude oil storage tank entered through a manway

3. B — A crust over a hidden void that can collapse when stood on

4. D — NOT safe — the oxygen level is deadly and the LEL reading cannot be trusted

5. A — Air-purge (ventilate) it and test the oxygen at all levels

6. C — Site Controller – overall control of work and SIMOPS; Area Authority – safe state of the plant

7. A — Performing Authority – controls the work party and toolbox talk; Area Operator – applies and verifies isolations

8. D — It is the last line of defence, used after other controls

9. C — Positive isolation (for example, blinding or disconnection)

10. B — It can be defeated by a signal, a fault, an operator action or a system restart

11. A — Barrier it as soon as the cover is removed and keep it clear of hoses and cables

12. C — A filter cannot create oxygen

13. D — Raise the alarm and activate the rescue plan — do not enter

14. B — It needs a hot work permit, continuous gas monitoring and a fire watch

15. A — Some mixtures give off toxic gas or heat (e.g. acid with sulphide gives H₂S)

16. C — Evacuate the space first, then investigate

17. B — Work stops and entrants leave; the permit is revalidated or reissued and the space re-tested before re-entry

18. D — Non-entry rescue using the retrieval system, planned before the permit is issued

19. A — After an incident or near miss involving confined space entry

20. C — No — only an authorised person may perform the role

21. D — Recognising pyrophoric scale, NORM scale and hazardous deposits

22. B — Verified at mobilisation, not accepted on assurance

23. C — Certified explosion-protected (Ex) equipment, preferably low-voltage or battery powered

24. D — A full body harness connected to a tripod or davit with a winch or self-retracting lifeline

25. A — They must be secured, inspected before use and kept clear so entrants can leave quickly